栈指针sp分析过程

sp寄存器，只是一个寄存器，并不在帧中有存储空间，而是永远指向最低的栈地址。并且可以显示其值，而这个地址在64位系统中为8位。

通过一个简单的函数，分析sp的一些原理跟机制。C语言表示：

int main2()

{

int a=100;

long long b=20000000000;

func(a,  b, t);

return 0;

}

汇编：

(gdb) disassemble /r main2

Dump of assembler code for function main2:

0x0000555555555684 <+0>:     1101                    addi    sp,sp,-32 #申请栈空间

0x0000555555555686 <+2>:     ec06                    sd      ra,24(sp)

0x0000555555555688 <+4>:     e822                    sd      s0,16(sp) #

0x000055555555568a <+6>:     1000                    addi    s0,sp,32#为什么要用栈帧寄存器呢？

0x000055555555568c <+8>:     06400793                li      a5,100

0x0000555555555690 <+12>:    fef42223                sw      a5,-28(s0) #给a赋值并压入栈

0x0000555555555694 <+16>:    009507b7                lui     a5,0x950

0x0000555555555698 <+20>:    2f978793                addi    a5,a5,761 # 0x9502f9

0x000055555555569c <+24>:    07ae                    slli    a5,a5,0xb #这个指令还不熟悉

0x000055555555569e <+26>:    fef43423                sd      a5,-24(s0) #给b赋值压入栈

=> 0x00005555555556a2 <+30>:    00002797                auipc   a5,0x2

0x00005555555556a6 <+34>:    99678793                addi    a5,a5,-1642 # 0x555555557038 <t>

0x00005555555556aa <+38>:    6398                    ld      a4,0(a5) #寄存器a4 指向全局指针t

0x00005555555556ac <+40>:    fe442783                lw      a5,-28(s0) #取a5的值100

0x00005555555556b0 <+44>:    863a                    mv      a2,a4 #a2指向？

0x00005555555556b2 <+46>:    fe843583                ld      a1,-24(s0)#a1为2000000000

0x00005555555556b6 <+50>:    853e                    mv      a0,a5#a0 为100

0x00005555555556b8 <+52>:    fadff0ef                jal     0x555555555664 <func>

0x00005555555556bc <+56>:    4781                    li      a5,0

0x00005555555556be <+58>:    853e                    mv      a0,a5

0x00005555555556c0 <+60>:    60e2                    ld      ra,24(sp)

0x00005555555556c2 <+62>:    6442                    ld      s0,16(sp)

0x00005555555556c4 <+64>:    6105                    addi    sp,sp,32

0x00005555555556c6 <+66>:    8082                    ret

思考？？

为什么要用栈帧寄存器呢？直接操作sp不一样吗？他们都能正确指向栈帧的地址哈。

- 保存上一个函数的现场

| 0x7ffffffff0d0，这是main2函数上级调用时sp指向的地址。 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0x7ffffffff0cf | 0x7ffffffff0ce | 0x7ffffffff0cd | 0x7ffffffff0cc | ra的栈内存 |
| 0x7ffffffff0cb | 0x7ffffffff0ca | 0x7ffffffff0c9 | 0x7ffffffff0c8 |  |
| 0x7ffffffff0c7 | 0x7ffffffff0c6 | 0x7ffffffff0c5 | 0x7ffffffff0c4 | s0的栈内存 |
| 0x7ffffffff0c3 | 0x7ffffffff0c2 | 0x7ffffffff0c1 | 0x7ffffffff0c0 |  |
| 0x7ffffffff0cf | 0x7ffffffff0be | 0x7ffffffff0bd | 0x7ffffffff0bc | 双字long long 型，b参数的 |
| 0x7ffffffff0bb | 0x7ffffffff0ba | 0x7ffffffff0b9 | 0x7ffffffff0b8 |  |
| 0x7ffffffff0b7 | 0x7ffffffff0b6 | 0x7ffffffff0b5 | 0x7ffffffff0b4 | int型a参数的 |
| 0x7ffffffff0b3 | 0x7ffffffff0b2 | 0x7ffffffff0b1 | 0x7ffffffff0b0 | 空字节对齐，4个字节 |

- 先对申请32个字节的栈空间。至于为什么是32个字节（不排除优化后这个字节数量会有变化）？

- 前面16个字节：

- 高8位保存，ra(x1)返回寄存器的8个字节

- 低8位保存，s0/fp(x8)栈帧寄存器的8个字节

- 后面12个字节：

- 前面8个字节，用来传参，存储一个long long类型数据，我认为可以理解为使用		了一个函数的临时变量，而临时变量是保存在栈帧当中的。

- 同理，后4个字节存储int型数据。

- 最面4个字节：

- 此处为空内容，为了8字节对齐吧。通过下面思考得来的。因为有的栈空间申请，只是向下要了16个字节。

思考，这4个字节有必要吗？

还是为了对齐占空间的存储空间？8字节对齐吧。呵呵，应该是如此了，因为据我观察，每次申请占空间，都是以8字节为最小单位的。

汇编分析——压缩指令c.addi

0x00005555555556d0 <+0>:     1101                    addi    sp,sp,-32

0x00005555555556d2 <+2>:     ec06                    sd      ra,24(sp)

0x00005555555556d4 <+4>:     e822                    sd      s0,16(sp)

0x00005555555556d6 <+6>:     1000                    addi    s0,sp,32

机器码：0x 1101

二进制表示：

0001 0001 0000 0001

000

1			#imm[1*****]

0001 0		#rd ----sp寄存器

000 00		#imm[1*****]----> 10 0000

01

因为偏移量只有6二进制位，并且是一个有符号的立即数，imm[5]，可以理解为符号位。永远要做符号扩展的，

而此处：

10 0000这个二进制做符号扩展：1110 0000刚好是32

思考？？

怎么对有符号的2进制的负数做十进制转换。

汇编分析——栈空间申请以及保存现场

0x7ffffffff0d0

7FFF FFFF F0C8

7FFF FFFF F0C0

7FFF FFFF F0B8

0x7ffffffff0b0

main2

addi    sp,sp,-32

sd      ra,24(sp) 0x7ffffffff0c8: 0x00005555555556d4

sd      s0,16(sp) 0x7ffffffff0c0: 0x00007ffffffff0e0

addi     s0,sp,32  s0             0x7ffffffff0d0

| 0x7ffffffff0d0，这是main2函数上级调用时sp指向的地址。 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0x7ffffffff0cf | 0x7ffffffff0ce | 0x7ffffffff0cd | 0x7ffffffff0cc | ra的栈内存 |
| 0x7ffffffff0cb | 0x7ffffffff0ca | 0x7ffffffff0c9 | 0x7ffffffff0c8 |  |
| 0x7ffffffff0c7 | 0x7ffffffff0c6 | 0x7ffffffff0c5 | 0x7ffffffff0c4 | s0的栈内存 |
| 0x7ffffffff0c3 | 0x7ffffffff0c2 | 0x7ffffffff0c1 | 0x7ffffffff0c0 |  |
| 0x7ffffffff0cf | 0x7ffffffff0be | 0x7ffffffff0bd | 0x7ffffffff0bc | 双字long long 型，b参数的 |
| 0x7ffffffff0bb | 0x7ffffffff0ba | 0x7ffffffff0b9 | 0x7ffffffff0b8 |  |
| 0x7ffffffff0b7 | 0x7ffffffff0b6 | 0x7ffffffff0b5 | 0x7ffffffff0b4 | int型a参数的 |
| 0x7ffffffff0b3 | 0x7ffffffff0b2 | 0x7ffffffff0b1 | 0x7ffffffff0b0 | 空字节对齐，4个字节 |

汇编分析——赋值，压栈sw

- 汇编li指令load immediate的缩写，相当于把100赋值给a5

- 2.汇编sw指令store word的缩写。汇编sw指令的意思是把a5的值压入s0-28为开始的4字节空间。

=> 0x000055555555568c <+8>:     li      a5,100

0x0000555555555690 <+12>:    sw      a5,-28(s0)

0x7ffffffff0b4: 0x0000000000000064

Int a的地址：

$2 = (int *) 0x7ffffffff0b4

这时候通过li跟sw两条汇编语句给，a的地址赋值100（0x64）

也就是堆内存初始化

汇编分析——赋值，压栈sd

直接源码分析就可。

0x0000555555555694 <+16>:    lui     a5,0x950

0x0000555555555698 <+20>:    addi    a5,a5,761 # 0x9502f9

0x000055555555569c <+24>:    slli    a5,a5,0xb

0x000055555555569e <+26>:    sd      a5,-24(s0)

同理这条语句给b赋值

(gdb) x /g 0x7ffffffff0d0-24

0x7ffffffff0b8: 0x00000004a817c800

(gdb) x /dg 0x7ffffffff0d0-24

0x7ffffffff0b8: 20000000000

(gdb) p &b

$3 = (long long *) 0x7ffffffff0b8

汇编分析——指针，全局数据初始化。

0x00005555555556a2 <+30>:    auipc   a5,0x2

0x00005555555556a6 <+34>:    addi    a5,a5,-1642 # 0x555555557038 <t>

0x00005555555556aa <+38>:    ld      a4,0(a5)

# 0x555555557038 <t>在全局数据区，

取数据区的数据给a4赋值，是一个数据区字符串

(gdb) p t

$8 = 0x5555555556e8 "ssssssssss\n"

(gdb) x t

0x5555555556e8: "ssssssssss\n"

(gdb) p &t

$9 = (char **) 0x555555557038 <t>

(gdb) i r a5 a4

a5             0x555555557038   93824992243768		#t的地址

a4             0x5555555556e8   93824992237288		#t指向的内容

(gdb)

全局变量，已经初始化过了。这里只是取t的地址赋值给a4，相当于指针赋值。

额

思考？？

全局数据怎么赋值的，还得再看看。

汇编分析——压缩指令c.addi

c.ld rd’, uimm(rs1’) 								x[8+rd’] = M[x[8+rs1’] + uimm][63:0]

双字加载 (Load Doubleword). RV64IC.

扩展形式为 ld rd, uimm(rs1), 其中 rd=8+rd’, rs1=8+rs1’.

011op   0 00im[--***---]  11 1rs' 00im[**------]  1 10rd'  00op

im[--***---]

im[**------]

0x00005555555556aa <main2+38>:       6398                    ld      a4,0(a5)

思考？？

压缩指令ld，发现它太短了，并且立即数貌似也没什么用，然后慢慢查资料得到下

面的那个表格跟链接地址。1. 得到下面的东西。2.全局字符串赋值

0110 0011 1001 1000

好变态的压缩指令，看不懂

011op   0 00im[--***---]  11 1rs' 00im[**------]  1 10rd'  00op

im[--***---]

im[**------]

110 ‘rd  = x14 a4

111 rs  a5

如何在RISC中为C.LD压缩指令取即时/偏移位？-腾讯云开发者社区-腾讯云

汇编分析——全局字符串赋值

# 0x555555557038 <t>在全局数据区，

取数据区的数据给a4赋值，是一个数据区字符串

(gdb) p t

$8 = 0x5555555556e8 "ssssssssss\n"

(gdb) x t

0x5555555556e8: "ssssssssss\n"

(gdb) p &t

$9 = (char **) 0x555555557038 <t>

(gdb) i r a5 a4

a5             0x555555557038   93824992243768		#t的地址

a4             0x5555555556e8   93824992237288		#t指向的内容

(gdb)

全局变量，已经初始化过了。这里只是取t的地址赋值给a4，相当于指针赋值。

额

思考？

指针，字符串，全局数据区是什么机制，为什么指针只给地址，不给内容，计算机怎么识别呢？

汇编分析——验证自己的学习点

思考？？

函数传参压栈，出栈的值。主要还要熟悉其过程，比如说指针，结构体，函数指针是怎么个传参法？

各个寄存器的值，临时变量的值，栈中的内容，地址是否对的上等等。。。。如下图

sp addr: 0x7ffffffff0d0 此刻假设为32，或理解为取相对地址。

| main中sp的addr | 0x7ffffffff0d0 | 0xfffff210 |
| --- | --- | --- |
| 28 | 存8个字节 | 0x5555555556d4 |
| 24暂存ra |  |  |
| 20 | 存8个字节 | 0x7ffffffff0e0 |
| 16暂存s0 |  |  |
| 12 | 存8个字节 0x7ffffffff0b8: 20000000000 |  |
| 8 -4(s0) |  |  |
| 4 - 28(s0) | 存4个字节0x7ffffffff0b4: 100 |  |
| 0 sp指向这个地址 | 0x7ffffffff0b0 | 0x00000000 |

0000000000000684 <main2>:

684:	1101                	addi	sp,sp,-32

686:	ec06                	sd	ra,24(sp)

688:	e822                	sd	s0,16(sp)

68a:	1000                	addi	s0,sp,32

68c:	06400793          	li	a5,100

690:	fef42223          	sw	a5,-28(s0)

694:	009507b7          	lui	a5,0x950

698:	2f978793          	addi	a5,a5,761 # 9502f9 <__global_pointer$+0x94daf9>

69c:	07ae                	slli	a5,a5,0xb

69e:	fef43423          	sd	a5,-24(s0)

6a2:	00002797          	auipc	a5,0x2

6a6:	99678793          	addi	a5,a5,-1642 # 2038 <t>

6aa:	6398                	ld	a4,0(a5)

6ac:	fe442783          	lw	a5,-28(s0)

6b0:	863a                	mv	a2,a4

6b2:	fe843583          	ld	a1,-24(s0)

6b6:	853e                	mv	a0,a5

6b8:	fadff0ef          	jal	664 <func>

6bc:	4781                	li	a5,0

6be:	853e                	mv	a0,a5

6c0:	60e2                	ld	ra,24(sp)

6c2:	6442                	ld	s0,16(sp)

6c4:	6105                	addi	sp,sp,32

6c6:	8082                	ret

=> 0x00005555555556d0 <main+8>: fb5ff0ef                jal     0x555555555684 <main2>

(gdb) i r sp

sp             0x7ffffffff0d0   0x7ffffffff0d0

sp             0x7ffffffff0b0   0x7ffffffff0b0

(gdb) i r

ra             0x5555555556d4   0x5555555556d4 <main+12>

sp             0x7ffffffff0b0   0x7ffffffff0b0

gp             0x555555557800   0x555555557800

tp             0x7ffff7fcf3c0   0x7ffff7fcf3c0

t0             0x7fffffffedf0   140737488350704

t1             0x7ffff7e752bc   140737352520380

t2             0xa46    2630

fp             0x7ffffffff0e0   0x7ffffffff0e0

s1             0x7ffffffff278   140737488351864

a0             0x1      1

a1             0x7ffffffff278   140737488351864

a2             0x7ffffffff288   140737488351880

a3             0x0      0

这个表格便于我们理解栈中的空间，跟sp指向的地址之间的一种关系

| 0x7ffffffff0d0，这是main2函数上级调用时sp指向的地址。 |  |  |  |
| --- | --- | --- | --- |
| 0x7ffffffff0cf | 0x7ffffffff0ce | 0x7ffffffff0cd | 0x7ffffffff0cc |
| 0x7ffffffff0cb | 0x7ffffffff0ca | 0x7ffffffff0c9 | 0x7ffffffff0c8 |
| 0x7ffffffff0c7 | 0x7ffffffff0c6 | 0x7ffffffff0c5 | 0x7ffffffff0c4 |
| 0x7ffffffff0c3 | 0x7ffffffff0c2 | 0x7ffffffff0c1 | 0x7ffffffff0c0 |
| 0x7ffffffff0cf | 0x7ffffffff0be | 0x7ffffffff0bd | 0x7ffffffff0bc |
| 0x7ffffffff0bb | 0x7ffffffff0ba | 0x7ffffffff0b9 | 0x7ffffffff0b8 |
| 0x7ffffffff0b7 | 0x7ffffffff0b6 | 0x7ffffffff0b5 | 0x7ffffffff0b4 |
| 0x7ffffffff0b3 | 0x7ffffffff0b2 | 0x7ffffffff0b1 | 0x7ffffffff0b0 |

100

0

0110 0

011 10

10